Windows CE 5.0 MCS7830 EPROM Utility User Manual

- EPROM Utility is a Command Based Tool which reads and writes the MAC Address from and to the Device (Revision B Device only).

- See the Usage Options and the Sample Output below.

Usage

EpromUtility r -Reads the MAC Address from the Device if the Device Instance ‘MOSUSBCE1’ is present.

EpromUtility w -Writes the MAC Address to the EPROM provided the 6 Bytes of MAC Address is Hex is passed.

Note: Please make sure that the Values given are Hex Data. (Both 0x50 and 50 are treated as Hex).

Sample Input/Output

>epromutility r

Adapter Found

MAC Address From EEPROM

0x 0  0x50  0xc2  0x1b  0x70  0x78

> epromutility r 1

Adapter Found

MAC Address From EEPROM\

0x 0  0x50  0xc2  0x1b  0x70  0x78

> epromutility w

Adapter Found

Wrong Number of Arguments: NumBytes 0 Required 6 Bytes for MAC Address

> epromutility w 00 50 c2 1b 70 99

Adapter Found

Programming MAC Address...

EPROM Programmed Successfully

> epromutility w 00

Adapter Found

Wrong Number of Arguments: NumBytes 1 Required 6 Bytes for MAC Address

> epromutility

USAGE::

1.To Read MAC Address - 'EpromUtility r'

2.Write MAC Address   - 'EpromUtility w 00 50 C2 1b 70 75'